Configuration of syslog server

To install the package 			#yum install rsyslog * -y

To edit the configuration file 			#vi /etc/rsyslog/rsyslog.conf

O/p 		To remove # from line num 15 16 19 20 		:wq (save)

To restart the syslog server 			#service rsyslog restart

To client side

To edit the configure				#vi /etc/rsyslog.conf

O/p  To end of the file place assign syslog server ip   192.168.0.48		:wq(save)

To restart the server 				#service rsyslog restart

To verify the port opening by issuing the following command	#telnet 192.168.0.105   514

To moitor the acivity 				#tailf  /var/log/message